Documentation for the Beacon Identifier program BCN.EXE

by Lyle Koehler, K0LR   Rt 1 Box 659  Aitkin, MN 56431

e-mail: k0lr@arrl.net

BCN.EXE is an ASCII-to-Morse converter tailored specifically for use

as a beacon identifier.  It can also be used for code practice or as a

keyboard keyer, but there are other PC programs that do a better job in

those departments.  This program was compiled with Microsoft

QuickBASIC 4.0 from the source code contained in the file BCN.BAS.

You can also run BCN.BAS directly under the BASIC interpreter

resident in most PCs.  However, I didn't optimize the software for speed

and relied on the QuickBASIC compiler to make up for my lack of

programming expertise.  As a result the BASIC version will start to bog

down at speeds much above 20 WPM on slower machines.  BCN.EXE,

on the other hand, should pump out a beacon identifier at code speeds

beyond the requirements of any sane experimenter.  To run either

BCN.EXE or BCN.BAS, the file BCN.CFG must reside within the same

drive and directory.  A startup BCN.CFG file is included with the

zipped program files.  You can easily tailor the BCN.CFG file to fit your

needs from within BCN.EXE.

This program can be activated automatically upon power-up, and will

keep repeating an identifier message until you interrupt it.  The PC

can be tucked in a corner of the shack and doesn't even need a

keyboard or monitor as long as you have access to another computer

to use for editing the BCN.CFG file.  If the program is run on a

computer with a keyboard and monitor, there are a number of options

you can invoke "on the fly".  You can change the code speed, enter

messages and store them to disk, read existing disk files, change

output ports, change the tone frequency for the speaker output or use

the computer as a keyboard keyer.  I've tested the program on an old

IBM PC and on a newer 386DX40 clone.  It should work on any IBM-

compatible clone with DOS 2.1 or later, but I can't guarantee it.  One

thing it won't do is work properly under Windows, even via the

Windows MS-DOS prompt.  You have to be in stand-alone DOS to run

this program.  The speaker output option may function in a Windows

environment but other output ports probably will not.

The code output can be directed to one of the serial or parallel ports or

to the PC's speaker, but not simultaneously to the speaker and an

output port.  When the COM ports are used, the "normal" output (logic

level high corresponding to the key-down condition) is the Data

Terminal Ready line (pin 20 on a DB-25 connector or pin 4 on a DB-9

connector).  An inverted output is available simultaneously on the

Request To Send line (DB-25 pin 4 or DB-9 pin 7).  Logic ground is pin

7 on a DB-25 connector or pin 5 on a DB-9.

Most line printer ports on IBM PCs and clones are DB-25 female

connectors.  Normal keying is provided on data lines 0 through 3 (pins

2-5) and inverted keying is available on data lines 4 through 7 (pins 6-

9).  Pins 18 through 25 on the line printer connectors are logic ground.

Keying of most solid-state transmitters (keying voltages under 20 volts

and currents under 20 mA) can be accomplished by connecting a 3K

resistor between the appropriate output line and the base of an NPN

transistor.  I recommend putting a diode between the emitter and base

with the cathode (bar end) tied to the base.  This keeps the -12 volt

swings on an RS-232 COM port from avalanching the base-emitter

junction.  The emitter of the transistor is tied to logic ground, and the

collector can be connected directly to the keying line of a typical solid-

state beacon transmitter.  An optoisolator or relay to isolate the keying

and ground lines of the transmitter and computer is strongly

recommended to keep static charges and other stray voltages from

zapping the computer.  Suggested interface circuits are contained in

the article "Using a PC as a Beacon Message Generator" on the author's

web site at http://www.qsl.net/k0lr/  The file PCINTFC.PCX also has this

information and can be viewed on programs such as Paintbrush.

When you run the program, the computer first spends one second

initializing its timing loops.  Then the program reads the BCN.CFG file,

which must be in the directory from which you started the program.  As

soon as the configuration is loaded, the computer displays the

message it's going to send and then begins putting out the message on

the specified port.  It also displays each character as it is sent.  The

ID string, code speed and output port selection are defined in the

BCN.CFG file, and the current configuration will be displayed prior to

sending the message.  If you haven't edited the ID string it will just be a

series of V's  The screen will also list these keyboard commands:

D	add or remove a long dash after the message

N       enter New message

C       Change code speed

K       allow Keyboard input

R       Read disk file

W       Write disk file

I       change Identifier string

A       Audible tone

X       inaudible tone

F       change Frequency

S       Save current configuration to disk

P       change output Port

Q       Quit to DOS

Some beacons operators like to send a long dash after the identifier

(this format is called dash after ID or DAID) as an aid in tuning or

for making signal strength messages. If you press D while the program

is running, the computer will ask if you want a long dash after the

message. Pressing anything but Y or y before hitting ENTER will turn

off the long dash. If you answer Y or y, the program will ask you to

enter the desired dash length in seconds.

Upon power-up the computer sends only the ID string stored in the

BCN.CFG file. You can insert an additional message in front of the ID

string by pressing N (New message) or R (Read disk file).  This

additional message will be wiped out if you exit the program although

you can store it to disk with the W command.

Press N to enter a new message from the keyboard.  With BCN.EXE

the total length of this message plus the ID string can be up to 32K

characters but must be entered in strings not more than 255 characters

long.  If you use BCN.BAS the message and ID strings are limited to

255 characters each.  You're safe if you stop before the end of the third

line on an 80-column display.  When you finish each string by pressing

the ENTER key the computer will ask if you want to add more text.  If

yes, press Y followed by the ENTER key; if no, just press ENTER.

Commas can't be included in the text string because the computer will

think you're trying to enter additional variables it hasn't asked for.  If

you want a comma in your message, use a backslash (\) or enclose the

whole message in quotes.  The program automatically inserts spaces

between pieces of the message, so don't press ENTER in the middle of

a word.  When you're done the computer displays the combined

message (what you just entered followed by the identifier string) and

then begins repeating the whole message in Morse code.  As each

character is sent it is also displayed on the screen.

To change the code speed, press C and then type in the new speed in

response to the prompt, followed by the ENTER key.  This program

does not send slow code in the Farnsworth method, in which the

character speed is kept at about 13 WPM and the spacing is

exaggerated as the speed is decreased.  The Farnsworth method is

preferred by most people for code practice and for QSOs but is usually

not desirable for beacon identification.  This program keeps the same

character-to-spacing ratio at all speeds.

The K key enters the keyboard keyer mode.  There isn't much of a

keyboard buffer so you can type only a few characters ahead of the

outgoing code.  For obvious reasons the various command keys are

not available in the keyboard keyer mode.  Pressing the ESCAPE key

will return the program to the normal mode and immediately resume

sending any previously entered message and the ID string.

Pressing R will allow you to read an existing text file from disk.  After

you enter the desired file name (including the path if not in the current

directory) and press the ENTER key, the computer will read and

display the file and then begin sending it.  Note: You do not enclose

the name in quotes as you would when loading a BASIC file.  The ID

message is tacked on to the end and becomes part of the total

message string.  BCN.EXE can handle disk files up to 32K bytes;

BCN.BAS is limited to files of 255 bytes or less.

You can save a message you've entered by pressing W.  The

computer will ask you for the file name, which can be any legal DOS

name.  Be careful not to use a name that already exists unless you

really want to overwrite it.  The program will not warn you of an existing

file name conflict.

Entering a new ID string is done in the same way you enter a new

message, except that you get there by pressing the I key.  The new ID

string will temporarily replace the old one but will be lost when you exit

the program unless the new configuration is saved as described below.

When the output is directed to the speaker, the A, X and F keys turn

the speaker on and off and let you change the tone frequency.

Actually the X key doesn't shut off the speaker but makes the "on" tone

the same as the "off" tone (32,767 Hz).

To redirect the output to a different port, press P.  You will be given a

choice of several standard port addresses, the speaker, or a user-

defined port address.  The standard addresses stored in the program

are 3FCH, 2FCH, 378H, 3BCH and 278H.  These correspond on many

PCs to COM1, COM2, LPT1, LPT2 and LPT3.  If the COM

port addresses look unfamiliar it's because they are the addresses for

the port's control register and are offset from the commonly-used port

address.  For example, the address for COM1's data buffer is 3F8H but

its control register is 3FCH.  LPT port addresses can also be confusing.

LPT1 is usually at 378H, but on some computers it's at 3BCH and the

address 378H is used for LPT2.  To be sure of the correct address you

have to consult the hardware documentation or use a diagnostic program

to show you the base address for the desired port on your computer.  In

the case of the COM ports you need to add 4 to the base address to

correct it for the offset.  A user-defined output port address must be

entered as a 3-character hexadecimal number.The program will also ask

you to enter the numbers you want written to the port to activate the

desired output lines. Suggested values are 1, 2 for COM ports and

15, 240 for LPT ports.  These numbers will key the DTR/RTS or LPT

data lines.  Undesirable things may happen if you direct the output to a

port that is already in use.  Example:  If you direct the output to a port

where you have a serial mouse connected, it will stun the mouse and

you will need to re-boot the computer to revive it.

When you have set up the ID string, output port, code speed and

speaker tone the way you want them, press S to save the configuration

to the BCN.CFG file.  The computer will display the current

configuration and ask for confirmation that you want to overwrite the

old BCN.CFG file.

Pressing Q will result in a prompt asking if you want to quit to DOS.

Entering Y and pressing the ENTER key will get you back to the DOS

prompt, even if you are running the BCN.BAS program.  If you're

running the BASIC version of the program and want to stop execution

without leaving BASIC, use CONTROL-BREAK instead of Q.

To run this program automatically upon startup on a computer with a

single floppy drive, you will need to have a diskette containing the DOS

system files including COMMAND.COM, along with the files BCN.EXE

and BCN.CFG.  Using the DOS format A: /S command will put the

necessary system files on a blank diskette.  Be sure the diskette you

start with doesn't contain any files you want to keep because the format

command will irrevocably erase any existing files.  After the diskette is

formatted, you can copy BCN.EXE and BCN.CFG to it.  The diskette

will also need to have an AUTOEXEC.BAT file with the command

needed to start the program -- in this case, just the letters  BCN

BCN.EXE and BCN.BAS both have to read a BCN.CFG file of the

proper format or the program will stop execution and give an error

message.  If for some reason your BCN.CFG file is deleted or

corrupted, you can go into BCN.BAS and disable program line 120,

which calls the subroutine that reads BCN.CFG.  Then you can run the

program using its internal defaults and create a new BCN.CFG file

using the S command described above.  You can also create a

configuration file on any text editor and save it as unformatted text as

BCN.CFG.  The required format is :  {ID string}, {speaker enable flag},

{speaker tone}, {WPM}, {port address}, {keydown variable}, {keyup

variable}, {long dash enable flag}, {dash length in seconds}  The default

BCN.CFG file shown below has an ID string of  " V  V  V", speaker is

enabled, tone frequency = 800 Hz, code speed = 12 WPM, port = COM2

(the address can be stated as &H2FC or its decimal equivalent of 764),

keydown variable = 1, keyup variable = 2, long dash is enabled,

dash length = 5 seconds.

Default BCN.CFG file:          " V V V",1,800,12,764,1,2,1,5

Note: If the speaker is enabled, the output port information is

disregarded by the program.  However, BCN.CFG has to see some

kind of numbers in the proper positions.

This program recognizes the following keyboard characters:

ABCDEFGHIJKLMNOPQRSTUVWXYZ 1234567890 . , ; ? / = - : ' ) \ + ]

[ & !

Letter and number keys and common punctuation marks are sent as

the usual Morse characters.  There are several special keys.

Backslash (\) is sent as a comma and must be substituted for commas

in messages you enter from within the program.  Other special keys

are: + (end of message), ] (end of work), [ (wait), & (ES), and ! (CQ).

A file called "TEST" is provided with the program.  Reading this file will

demonstrate all the keyboard characters which are available for

conversion to Morse.

73 and good DX!